- Prerequisites: The CAN card has been configured and is communicating normally with the chassis.

- ‌Compilation‌

- 1.1 Open a terminal and navigate to the FW-max-ros1 directory.

- 1.2 Enter the command catkin_make and wait for compilation to complete.

- ‌Execution‌

- 2.1 Open a terminal, navigate to the FW-max-ros1 directory, and enter the following commands sequentially:

- source devel/setup.bash

- roslaunch yhs_can_control yhs_can_control.launch

- 2.2 If the output displays >>open can device success!, the initialization was successful.

- Testing‌

3.1 Before testing, it is recommended to lift the chassis or set the speed command to a very low value.

- 3.2 Open a terminal, navigate to the FW-max-ros1 directory, and enter:

- source devel/setup.bash

- rostopic echo /ctrl_fb

- 3.3 If the feedback data refreshes continuously, the ROS driver package is operating normally.

- 3.4 ‌Motion Control‌

- 3.4.1 Open a terminal, navigate to the directory, and enter:

- source devel/setup.bash

- 3.4.2 Enter the following command (do not press Enter yet):

- rostopic pub -r 100 /ctrl_cmd

- 3.4.3 Use Tab to autocomplete the remaining fields. Input the gear, linear speed, or angular speed (note: angular speed is in degrees per second, not radians per second). Press Enter to execute. Switch the remote to automatic mode. The CAN card's red and green lights should alternate, indicating chassis movement.

Note: When opening a new terminal to monitor chassis data, always set the environment variable first:

source devel/setup.bash

‌Published Topics:

Frequency: Must exceed 50 Hz

Topic name: /ctrl_cmd

Topic type: yhs_can_msgs/ctrl_cmd

Topic variables:

uint8 ctrl_cmd_gear                 Target gear

float32 ctrl_cmd_x_linear             Target vehicle X-axis linear velocity

float32 ctrl_cmd_z_angular            Target vehicle Z-axis angular velocity

float32 ctrl_cmd_y_linear             Target vehicle Y-axis linear velocity

Topic name: /io_cmd

Topic type: yhs_can_msgs/io_cmd

Topic variables:

bool io_cmd_lamp_ctrl             Lamp control

bool io_cmd_unlock               Safety parking unlock switch

bool io_cmd_low_power_enable     Low consumption mode switch

bool io_cmd_lower_beam_headlamp  Lower beam switch

bool io_cmd_upper_beam_headlamp  Upper beam switch

uint8 io_cmd_turn_lamp           Steering lamp switch

bool io_cmd_braking_lamp         Braking lamp switch

bool io_cmd_clearance_lamp       Clearance lamp switch

bool io_cmd_fog_lamp             Fog lamp switch

bool io_cmd_speaker              Speaker switch

uint8 io_cmd_low_power_ratio      Low consumption mode triggering ratio

Topic name: /motor_cmd

Topic type: yhs_can_msgs/motor_cmd

Topic variables:

bool motor_cmd_drive_enable_lf      Left front drive motor control switch

bool motor_cmd_drive_enable_lr      Left rear drive motor control switch

bool motor_cmd_drive_enable_rf      Right front drive motor control switch

bool motor_cmd_drive_enable_rr      Right rear drive motor control switch

bool motor_cmd_steering_enable_lf    Left front steering motor control switch

bool motor_cmd_steering_enable_lr    Left rear steering motor control switch

bool motor_cmd_steering_enable_rf    Right front steering motor control switch

bool motor_cmd_steering_enable_rr    Right rear steering motor control switch

bool motor_cmd_power_restart        Motors power restart

Topic name: /steering_ctrl_cmd

Topic type: yhs_can_msgs/steering_ctrl_cmd

Topic variables:

uint8 ctrl_cmd_gear                Target gear

float32 steering_ctrl_cmd_velocity    Target vehicle steering wheel’s velocity

float32 steering_ctrl_cmd_steering    Target vehicle steering wheel’s angle

Subscribed Topic:

Topic name: /ctrl_fb

Topic type: yhs_can_msgs/ctrl_fb

Topic variables:

uint8 ctrl_fb_gear                 Current gear position feedback

float32 ctrl_fb_x_linear             Current vehicle X-axis linear velocity feedbacck

float32 ctrl_fb_z_angular           Current vehicle Z-axis angular velocity feedbacck

float32 ctrl_fb_y_linear            Current vehicle Y-axis linear velocity feedbacck

Topic name: /steering_ctrl_fb

Topic type: yhs_can_msgs/steering_ctrl_fb

Topic variables:

uint8 steering_ctrl_fb_gear                Current gear position feedback

float32 steering_ctrl_fb_velocity            Current vehicle steering wheel’s velocity feedback

float32 steering_ctrl_fb_steering            Current vehicle steering wheel’s angle feedback

Topic name: /motor_fb

Topic type: yhs_can_msgs/motor_fb

Topic variables:

bool motor_cmd_drive_enable_lf      Left front drive motor control switch feedback

bool motor_cmd_drive_enable_lr      Left rear drive motor control switch feedback

bool motor_cmd_drive_enable_rf      Right front drive motor control switch feedback

bool motor_cmd_drive_enable_rr      Right rear drive motor control switch feedback

bool motor_cmd_steering_enable_lf    Left front steering motor control switch feedback

bool motor_cmd_steering_enable_lr    Left rear steering motor control switch feedback

bool motor_cmd_steering_enable_rf     Right front steering motor control switch feedback

bool motor_cmd_steering_enable_rr     Right rear steering motor control switch feedback

bool motor_cmd_power_restart         Motors power restart feedback

Topic name: /io_fb

Topic type: yhs_can_msgs/io_fb

Topic variables:

bool io_fb_lamp_ctrl               Lamp control authority status feedback

bool io_fb_unlock                 Safety parking unlock status feedback

bool io_fb_lower_beam_headlamp   Low beam headlamp switch status feedback

bool io_fb_upper_beam_headlamp   Upper beam headlamp switch status feedback

int8 io_fb_turn_lamp               Turn signal switch status feedback

bool io_fb_braking_lamp            Braking lamp switch status feedback

bool io_fb_clearance_lamp          Clearance lamp switch status feedback

bool io_fb_fog_lamp               Fog lamp switch status feedback

bool io_fb_speaker                Speaker switch status feedback

uint8 io_fb_low_power_ratio        Low power trigger ratio feedback

bool io_fb_fl_impact_sensor         Front left impact sensor switch status feedback

bool io_fb_fm_impact_sensor        Front middle impact sensor switch status feedback

bool io_fb_fr_impact_sensor         Front right impact sensor switch status feedback

bool io_fb_rl_impact_sensor         Rear left impact sensor switch status feedback

bool io_fb_rm_impact_sensor        Rear middle impact sensor switch status feedback

bool io_fb_rr_impact_sensor         Rear right impact sensor switch status feedback

bool io_fb_fl_drop_sensor           Front left drop sensor status feedback

bool io_fb_fm_drop_sensor          Front middle drop sensor status feedback

bool io_fb_fr_drop_sensor           Front rear drop sensor status feedback

bool io_fb_rl_drop_sensor           Rear left drop sensor status feedback

bool io_fb_rm_drop_sensor          Rear middle drop sensor status feedback

bool io_fb_rr_drop_sensor           Rear right drop sensor status feedback

bool io_fb_estop                   Emergency stop switch status feedback

bool io_fb_joypad_ctrl              Remote controller status feedback

bool io_fb_charge_state             Charging station status feedback

bool io_fb_charger_sign              Manual charger signal feedback

bool io_fb_joypad_first              Remote controller first power-on status feedback

bool io_fb_joypad_online            Remote controller signal online status feedback

Topic name: /front_angle_fb

Topic type: yhs_can_msgs/front_angle_fb

Topic variables:

float32 front_angle_fb_l              Left front servo joint angle feedback

float32 front_angle_fb_r              Right front servo joint angle feedback

Topic name: /rear_angle_fb

Topic type: yhs_can_msgs/rear_angle_fb

Topic variables:

float32 rear_angle_fb_l               Left rear servo joint angle feedback

float32 rear_angle_fb_r               Right rear servo joint angle feedback

Topic name: /lf_wheel_fb

Topic type: yhs_can_msgs/lf_wheel_fb

Topic variables:

float32 lf_wheel_fb_velocity           Current left front wheel speed feedback

int32 lf_wheel_fb_pulse               Current left front wheel pulse count feedback

Topic name: /lr_wheel_fb

Topic type: yhs_can_msgs/lr_wheel_fb

Topic variables:

float32 lr_wheel_fb_velocity        Current left rear wheel speed feedback

int32 lr_wheel_fb_pulse            Current left rear wheel pulse count feedback

Topic name: /rf_wheel_fb

Topic type: yhs_can_msgs/rf_wheel_fb

Topic variables:

float32 rf_wheel_fb_velocity        Current right front wheel speed feedback

int32 rf_wheel_fb_pulse            Current right front wheel pulse count feedback

Topic name: /rr_wheel_fb

Topic type: yhs_can_msgs/rr_wheel_fb

Topic variables:

float32 rr_wheel_fb_velocity           Current right rear wheel speed feedback

int32 rr_wheel_fb_pulse              Current right rear wheel pulse count feedback

Topic name: /bms_flag_fb

Topic type: yhs_can_msgs/bms_flag_fb

Topic variables:

uint8 bms_flag_fb_soc                  Current remaining battery percentage

bool bms_flag_fb_single_ov              Individual cell overvoltage protection

bool bms_flag_fb_single_uv              Individual cell undervoltage protection

bool bms_flag_fb_ov                    Battery pack overvoltage protection

bool bms_flag_fb_uv                    Battery pack undervoltage protection

bool bms_flag_fb_charge_ot              Charging overtemperature protection

bool bms_flag_fb_charge_ut              Charging undertemperature protection

bool bms_flag_fb_discharge_ot            Discharging overtemperature protection

bool bms_flag_fb_discharge_ut            Discharging undertemperature protection

bool bms_flag_fb_charge_oc              Charging overcurrent protection

bool bms_flag_fb_discharge_oc            Discharging overcurrent protection

bool bms_flag_fb_short                  Short circuit protection

bool bms_flag_fb_ic_error                Front-end detection IC error

bool bms_flag_fb_lock_mos               Software locked MOS

bool bms_flag_fb_charge_flag             Charging flag bit

bool bms_flag_fb_heating_flag             Heating flag bit

float32 bms_flag_fb_hight_temperature     Current battery highest temperature

float32 bms_flag_fb_low_temperature      Current battery lowest temperature

Topic name: /bms_fb

Topic type: yhs_can_msgs/bms_fb

Topic variables:

float32 bms_fb_voltage             Current battery voltage

float32 bms_fb_current             Current battery current

float32 bms_fb_remaining_capacity   Current battery remaining capacity

Topic name: /ultrasonic

Topic type: yhs_can_msgs/ultrasonic

Topic variables:

uint16 front_left

uint16 front_right

uint16 rear_left

uint16 rear_right

uint16 left_front

uint16 left_rear

uint16 right_front

uint16 right_rear

Ultrasonic data topic, please refer to the launch file for specific calibration

Topic name: /error_fb

Topic type: yhs_can_msgs/error_fb

Topic variables:

uint8 error_fb_level             Fault level

uint8 error_fb_device_type       Fault component type

uint8 error_fb_devive_id         Fault component id

uint8 error_fb_emergency_code   Fault code

uint16 error_fb_register_code     Error register

Topic name: /drive_motor_current_fb

Topic type: yhs_can_msgs/drive_motor_current_fb

Topic variables:

float32 drive_motor_current_fb_lf           Left front drive motor current value

float32 drive_motor_current_fb_lr           Left rear drive motor current value

float32 drive_motor_current_fb_rf           Right front drive motor current value

float32 drive_motor_current_fb_rr           Right rear drive motor current value

bool drive_motor_oc_flag_fb_lf              Left front drive motor stall flag

bool drive_motor_oc_flag_fb_lr              Left rear drive motor stall flag

bool drive_motor_oc_flag_fb_rf              Right front drive motor stall flag

bool drive_motor_oc_flag_fb_rr              Right rear drive motor stall flag

Topic name: /steering_motor_current_fb

Topic type: yhs_can_msgs/steering_motor_current_fb

Topic variables:

float32 steering_motor_current_fb_lf             Left front steering motor current value

float32 steering_motor_current_fb_lr             Left rear steering motor current value

float32 steering_motor_current_fb_rf             Right front steering motor current value

float32 steering_motor_current_fb_rr              Right rear steering motor current value

bool steering_motor_oc_flag_fb_lf                Left front steering motor stall flag

bool steering_motor_oc_flag_fb_lr                Left rear steering motor stall flag

bool steering_motor_oc_flag_fb_rf                Right front steering motor stall flag

bool steering_motor_oc_flag_fb_rr                Right rear steering motor stall flag